Assignment-3

Task:1

Problem-01:

A bit stream 1101011011 is transmitted using the standard CRC method. The generator polynomial is x4+x+1. What is the actual bit string transmitted?

CODE :

#include <iostream>

#include <string>

#include <cmath>

#include <algorithm>

using namespace std;

#define int long long int

// Function to convert integer to binary string

string toBin(int num) {

// Handle case when number is 0

if (num == 0) return "0";

string bin = "";

while (num) {

// Append '1' or '0' based on least significant bit

bin += (num & 1) ? '1' : '0';

// Shift right to process next bit

num = num >> 1;

}

// Reverse string since bits were added in reverse order

reverse(bin.begin(), bin.end());

return bin;

}

// Function to convert binary string to decimal integer

int toDec(string bin) {

int n = bin.size();

// Handle empty string

if (n == 0) return 0;

int num = 0;

for (int i = 0; i < n; i++) {

if (bin[i] == '1') {

// Compute power of 2 for each '1' in binary string

num += 1 << (n - i - 1);

}

}

return num;

}

// Function to compute CRC and print remainder and codeword

void CRC(string data, string key) {

int n = key.length();

if (n == 0) {

cout << "Error: Key cannot be empty" << endl;

return;

}

// Convert binary strings to decimal integers

// Generator polynomial (key)

int gen = toDec(key);

// Original data

int code = toDec(data);

// Append (n - 1) zeros to the data to make space for CRC bits

int dividend = code << (n - 1);

// Calculate the position to start XOR (most significant bit position)

int shft;

while ((shft = (int)log2(dividend) - n + 1) >= 0) {

// Extract top 'n' bits of dividend, XOR with generator polynomial

int rem = (dividend >> shft) ^ gen;

// Replace top bits in dividend with XOR result (remainder)

dividend = (dividend & ((1 << shft) - 1)) | (rem << shft);

}

// Final codeword is the original data with the remainder appended

int codeword = (code << (n - 1)) | dividend;

// Print results

cout << "Remainder: " << toBin(dividend) << endl;

cout << "Codeword : " << toBin(codeword) << endl;

}

signed main() {

string data = "1101011011";

string key  = "10011";

CRC(data, key);

return 0;

}

OUTPUT :

Remainder: 1110

Codeword : 11010110111110

Task:2

Problem-02:

A bit stream 10011101 is transmitted using the standard CRC method. The generator polynomial is x3+1.

INPUT:

#include <bits/stdc++.h>

using namespace std;

// Performs bitwise XOR between two binary strings (a and b)

string findXor(string a, string b) {

int n = b.length();

string result = "";

// Compare each bit (skip first bit as per CRC standard)

for (int i = 1; i < n; i++) {

if (a[i] == b[i])

result += "0";

else

result += "1";

}

return result;

}

// Performs Modulo-2 division (CRC division algorithm)

string mod2div(string dividend, string divisor) {

int n = dividend.length();

int pick = divisor.length();

string tmp = dividend.substr(0, pick); // Initial window

while (pick < n) {

if (tmp[0] == '1')

// XOR with divisor and bring down next bit

tmp = findXor(divisor, tmp) + dividend[pick];

else

// XOR with zeros and bring down next bit

tmp = findXor(string(pick, '0'), tmp) + dividend[pick];

pick++;

}

// Final XOR step

if (tmp[0] == '1')

tmp = findXor(divisor, tmp);

else

tmp = findXor(string(pick, '0'), tmp);

return tmp;

}

// Appends CRC remainder to the original data

string encodeData(string data, string key) {

int n = key.length();

string paddedData = data + string(n - 1, '0'); // Append n-1 zeros

string remainder = mod2div(paddedData, key);

return data + remainder; // Return data + CRC

}

// Checks if received data has errors (remainder = 0)

int receiver(string code, string key) {

string remainder = mod2div(code, key);

return (remainder.find('1') == string::npos) ? 1 : 0;

}

int main() {

string data = "100100";

string key = "1101";

cout << "Sender Side" << endl;

cout << "Data: " << data << endl;

cout << "Key: " << key << endl;

string code = encodeData(data, key);

cout << "Encoded Data: " << code << endl << endl;

cout << "Receiver Side" << endl;

if (receiver(code, key))

cout << "Data is correct (No errors detected)" << endl;

else

cout << "Data is incorrect (Error detected)" << endl;

return 0;

}

OUTPUT :

Sender Side

Data: 100100

Key: 1101

Encoded Data: 100100001

Receiver Side

Data is correct (No errors detected)

- What is the actual bit string transmitted?

- Suppose the third bit from the left is inverted during transmission. How will receiver detect this error?

INPUT:

#include <iostream>

#include <string>

#include <cmath>

#include <algorithm>

using namespace std;

#define int long long int

// Function to convert integer to binary string

string toBin(int num) {

// Handle case when number is 0

if (num == 0) return "0";

string bin = "";

while (num) {

// Append '1' or '0' based on least significant bit

bin += (num & 1) ? '1' : '0';

// Shift right to process next bit

num = num >> 1;

}

// Reverse string since bits were added in reverse order

reverse(bin.begin(), bin.end());

return bin;

}

// Function to convert binary string to decimal integer

int toDec(string bin) {

int n = bin.size();

// Handle empty string

if (n == 0) return 0;

int num = 0;

for (int i = 0; i < n; i++) {

if (bin[i] == '1') {

// Compute power of 2 for each '1' in binary string

num += 1 << (n - i - 1);

}

}

return num;

}

// Function to compute CRC and print remainder and codeword

void CRC(string data, string key) {

int n = key.length();

if (n == 0) {

cout << "Error: Key cannot be empty" << endl;

return;

}

// Convert binary strings to decimal integers

// Generator polynomial (key)

int gen = toDec(key);

// Original data

int code = toDec(data);

// Append (n - 1) zeros to the data to make space for CRC bits

int dividend = code << (n - 1);

// Calculate the position to start XOR (most significant bit position)

int shft;

while ((shft = (int)log2(dividend) - n + 1) >= 0) {

// Extract top 'n' bits of dividend, XOR with generator polynomial

int rem = (dividend >> shft) ^ gen;

// Replace top bits in dividend with XOR result (remainder)

dividend = (dividend & ((1 << shft) - 1)) | (rem << shft);

}

// Final codeword is the original data with the remainder appended

int codeword = (code << (n - 1)) | dividend;

// Print results

cout << "Remainder: " << toBin(dividend) << endl;

cout << "Codeword : " << toBin(codeword) << endl;

}

signed main() {

string data = "10111101";

string key  = "1001";

CRC(data, key);

return 0;

}

OUTPUT :

Remainder: 10

Codeword : 10111101